Supplementary Data A – Calculating Evaporation & Harvesting potential as % of Mean annual load

Supplementary Data B – MUSIC Limitation modelling TSS below (12mg/l)

Supplementary Data C - Exfiltration rate impacts

Supplementary Data D - The calculations used to roughly estimate the cooling potential of the NWD street scenario.

Supplementary Data E – The calculations used to estimate the cooling potential of the WSUD scenario, including: A) Estimation of the impact of WSUD bioswale evaporation on cooling of street during summer; and B) Estimation of bioswale cooling combined with shade cooling estimate from urban heat modelling at the site.

Supplementary Data F – The master list of native vegetation species that was input into BAU, WSUD, and NWD designs in QGIS for analysis in the Vegetation Assessment Tool.

Supplementary Data G – A more detailed methodology of how Vegetation Assessment Tool metrics were assessed.

Supplementary Data H. Limitations of tools noted within method due to lack of integration.

Can be found at Repository:

Supplementary Data B. MUSICX Issue with higher output concentration in ponds:

MUSIC uses the following equation to calculate the outflow concentration from the pond. The default k and C* for TSS are 400 (m/yr) and 12 (mg/l), respectively.  12 (mg/l) means the minimum concentration from the outflow is about 12 mg/l. These parameter values are based on the previous study.

(C~out~ - C*) / (C~in~ - C*) = e –k/q  where:

C* = background concentration,

C~in~ = input concentration,

C~out~ = output concentration,

k = rate constant, and

q = hydraulic loading (flow rate per surface area) of the storage.

Generally, MUSIC is used to simulate the “dirty” stormwater treatment (at least concentration >12 mg/l for TSS ) . Other devices using K and C have similar requirements.

In this case, the stormwater quality to wetlands in TSS is “too” good sometimes. The results at PS_1_Perm show that the treated stormwater after the pond has the worst values in TSS concentration sometimes. (If the inflow concentration is less than 12, then the outflow concentration becomes 12).

Supported in consultation with eWater support.

Supplementary Data C. Exfiltration rate impacts

### Site-specific goals

It is important to tailor performance targets to site-specific goals. While groundwater recharge and infiltration are typically an important metric to evaluate in urban sponge-city and water management approaches, the infiltration of water into the sodic clay soils of the site was identified as a risk to the stability of the street, and therefore, an inappropriate goal for site safety52. Furthermore, the pre-development water balance of the landscape requires a low infiltration and high evapotranspiration rate38. Therefore, interaction between surface and groundwaters was reduced by lining the bioswale and ponds in MUSICX modelling. We also assumed an exfiltration rate of 0mm/hr, as per MUSIC guidelines in absence of a geotechnical visit53. While the NWD scenario did perform the best across all water metrics, the runoff reductions scores were lower for all scenarios in comparison to the goal of 81%38. Comparing the exfiltration of 0mm/hr to an alternate estimate of 1.8mm/hr (for highly sodic soils)54, the lower exfiltration rate may explain why water quality and runoff reduction across all scenarios was less than typical for sites with higher known exfiltration. Nevertheless, 0mm/hr was appropriate for our knowledge of the site.

Exfiltration rate 0mm/hr vs 1.8mm/hr:

Supplementary Data D. The calculations used to roughly estimate the cooling potential of the NWD street scenario. Calculations were made to roughly estimate the impact on potential street cooling of: A) The specific evaporation (L/day) of the pond systems in summer; B) The impact of NWD pond evaporation in summer; C) The impact of NWD bioswale evaporation during summer; D) The Combined impact of bioswale and pond cooling with NWD tree shading estimate of 0.4oC (55% canopy coverage from small and medium-sized trees54).

## A)  Calculation of the specific evaporation (L/day) of the pond systems in summer:

- Obtain Class A pan evaporation data from the nearest weather station from BOM (Bureau of Meteorology, “Evaporation (Pan)” under "Weather Data Services"  Nearest station: Kilmore Gap or Melbourne Airport.

- Use class A pan evaporation method:

- Find average daily pan evaporation (E_pan), e.g. ~mm/day in summer

- Multiply by pan coefficient (Kp): 0.7–0.85 (typical value: 0.75)

- Multiply by pond surface area

#### Formula:

Epond=Epan×Kp
Evaporated Volume (m³/day)=Epond(mm)1000×A(m2)

Data sources & assumptions

- Monthly ETo (mm/day) — from Bureau of Meteorology for Wallan (Kilmore Gap; ETo values typical for this region).

- Pond coefficient: Kc=0.8 (same for both ponds)

- Pond surface areas:

  - Pond A (20 m³): 24.5 m²

  - Pond B (15 m³): 19.5 m²

Calculation method for each pond & season:

- ETo volume over pond surface = ETo × area /1000 → m³/day

- Pond evaporation = Kc×ETomm/day → convert similarly to m³/day

- Evap loss as % of ETo = (Evap pond volume ÷ ETo volume) × 100% (should equal Kc×100%=80% in all cases, since we assume same coefficient and full exposure)

Supplementary Tables A&B. Estimates of the evaporation of ponds per day during summer for Pond System 1 & 2 of the NWD street.

| A.Pond System 1 (20 m³, area 24.5 m²) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Season | ETo (mm/day) | Pond Evap (mm/day) =0.8×ETo | ETo volume (m³/day) | Evap loss vol (m³/day) | Litres/day | % of ETo |
| Summer | ~5.0 | ~4.0 | 5.0/1000×24.5= 0.1225 m³ | 4.0/1000×24.5= 0.0980 m³ | 98 L/day | 80% |
| Autumn | ~3.5 | ~2.8 | 3.5/1000×24.5= 0.0858 m³ | 2.8/1000×24.5= 0.0686 m³ | 68.6 L/day | 80% |
| Winter | ~1.5 | ~1.2 | 1.5/1000×24.5= 0.0368 m³ | 1.2/1000×24.5= 0.0294 m³ | 29.4 L/day | 80% |
| Spring | ~3.5 | ~2.8 | same as Autumn: 0.0858 m³ | 0.0686 m³ | 68.6 L/day | 80% |

| B.Pond System 2 (15 m³, area 19.5 m²) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Season | ETo (mm/day) | Pond Evap (mm/day) | ETo volume (m³/day) | Evap loss vol (m³/day) | Litres/day | % of ETo |
| Summer | ~5.0 | ~4.0 | 5.0/1000×19.5= 0.0975 m³ | 4.0/1000×19.5= 0.0780 m³ | 78 L/day | 80% |
| Autumn | ~3.5 | ~2.8 | 3.5/1000×19.5= 0.0683 m³ | 2.8/1000×19.5= 0.0546 m³ | 54.6 L/day | 80% |
| Winter | ~1.5 | ~1.2 | 1.5/1000×19.5= 0.0293 m³ | 1.2/1000×19.5= 0.0234 m³ | 23.4 L/day | 80% |
| Spring | ~3.5 | ~2.8 | same as Autumn: 0.0683 m³ | 0.0546 m³ | 54.6 L/day | 80% |

## Estimating the impact of NWD pond evaporation on cooling of street during summer:

## Pond System 1 calculations:

- Latent heat of vaporisation:

Evaporated water removes energy in the form of latent heat of vaporisation.

Q=m⋅Lv

- Q= heat removed (in joules)

- m= mass of water evaporated (kg) = volume (litres)

- Lv= latent heat of vaporisation ≈ 2,450 kJ/kg (at ~20°C)

- Pond System 1 evaporates 98 litres/day in summer = 98 kg/day

- Energy removed:

Q=98×2450=240,100 kJ/day

Or in watts (power equivalent):

240,100 kJ/day=240,100×100086400≈2,778 W≈2.8 kW

~2.8 kW of heat is being removed from the local area continuously due to pond evaporation.

- Volume of Air Affected

Air volume = V=area affected×height

2400m² area of street × 5m high = 12,000 m³

Specific heat capacity of air: c=1.005 kJ/kg\cdotpK

Air density: ρ=1.2 kg/m³

Total heat capacity of air=ρ⋅V⋅c=1.2×12000×1.005=14472 kJ/K

Energy removed = 240,100 kJ/day

Divided by total heat capacity:

ΔT=Qρ⋅V⋅c=240,10014472≈16.59 K

…Estimate of the heat removed by Pond System 1 from the entire street to a height of 5m = 16.59 K

## Pond system 2 calculations:

- Latent heat of vaporisation:

Evaporated water removes energy in the form of latent heat of vaporisation.

Q=m⋅Lv

- Q= heat removed (in joules)

- m= mass of water evaporated (kg) = volume (litres)

- Lv= latent heat of vaporisation ≈ 2,450 kJ/kg (at ~20°C)

- Pond System 2 evaporates ~78 L/day in summer= 98 kg/day

- Energy removed:

Q=78×2450=191100kJ/day

Or in watts (power equivalent):

240,100 kJ/day=191100×100086400≈2211 W≈2.2 kW

~2.2 kW of heat is being removed from the local area continuously due to pond evaporation.

- Volume of Air Affected

Air volume = V=area affected×height

2400m² area of street × 5m high = 12,000 m³

Specific heat capacity of air: c=1.005 kJ/kg\cdotpK

Air density: ρ=1.2 kg/m³

Total heat capacity of air=ρ⋅V⋅c=1.2×12000×1.005=14472 kJ/K

Energy removed = 240,100 kJ/day

Divide by total heat capacity:

ΔT=Qρ⋅V⋅c=19110014472≈13.20 K

…Estimated heat removed by Pond system 2 from the entire street (150mx16m) to a height of 5m = 13.20 K

## Impact of Pond Evaporation on Street Temperature

| Factor | Estimate / Comment |
| --- | --- |
| Heat removed by Pond System 1 (summer) | - ~2.8 kW continuous (via latent heat) - 240,100 kJ/day - 16.59K/day to a height of 5m on (150x16m street) |
| Heat removed by Pond System 2 (summer) | - ~2.2 kW continuous (via latent heat) - 240,100 kJ/day 13.20K/day to a height of 5m on (150x16m street) |
| Local air volume influenced (for each) | ~300 m³ (approx. depending on wind) |

## Converting heat reduction to temperature reduction oC:

Pond System 1 conditions:

- Pond 1 latent cooling power = 2.8 kW ( = 2,800 J/s)

- Street area influenced = 2,400 m²

- Mixing height = 5 m

- Local air “reach” ≈ 300 m (this is consistent with using the 2,400 m² footprint)

- Location near Kilmore Gap (used only to anchor climate context)

## Estimating the equivalent air temperature reduction (ΔT) - assuming latent cooling is uniformly mixed through that air volume:

## 1. Calculating air volume being cooled

Vair=A×h=2400 m2×5 m=12,000 m3

## 2. Calculating heat capacity of that air volume

- Air density ρ=1.2 kg m-3

- Specific heat of air cp=1005 J kg-1K-1

C=Vair×ρ×cp
C=12,000×1.2×1005
C≈14,472,000 J K-1

14.5 MJ to change this air volume by 1°C.

## 3. Converting latent cooling power into temperature change rate

dTdt=PC
dTdt=2,80014,472,000=1.93×10-4 K s-1

Converting to per hour:

1.93×10-4×3600=0.695 K h-1

Kelvin degrees/hr  Celsius/hr cooling =0.695K h−1≈0.70K h−1≈0.70∘C h−1

Pond 1 alone provides a theoretical cooling rate of ~0.7°C per hour (for the 5 m deep, 2,400 m² street air volume).

Note: Field and modelling studies typically find only 20–40% of the theoretical cooling persists as a measurable air-temperature signal.

Applying that realism factor:

0.7×0.2 to 0.7×0.4=0.14 to 0.28∘C per hour

Over a hot afternoon with sustained evaporation (2–4 hours) Pond System 1 cooling:

~0.3∘C to 1.1∘C

## Converting heat reduction to temperature reduction oC (Pond System 2):

Pond System 2 conditions:

- Street area influenced = 2,400 m²

- Mixing height = 5 m

- Air volume = 12,000 m³

- Air density = 1.2 kg/m³

- Specific heat of air = 1005 J/kg·K

## 1. Heat capacity of affected air volume (same as before)

C=12,000×1.2×1005=14,472,000 J K-1

## 2. Temperature change rate for Pond System 2

P=2.2 kW=2,200 J s-1
dTdt=2,20014,472,000=1.52×10-4 K s-1

Convert to hourly:

1.52×10-4×3600=0.55 K h-1

Kelvin degrees/hr  Celsius/hr cooling =0.55K h−1≈0.55K h−1≈0.55∘C h−1

≈ 0.55 °C per hour

## Real-world effective cooling (20–40% persistence):

0.55×0.2→0.11∘C h-1
0.55×0.4→0.22∘C h-1

## Pond System 2 cooling over a 3–4 hour summer afternoon:

~0.3∘C to 0.9∘C

## Estimated Cooling from Pond System 1&2:

- Pond 1 effective rate ~0.14–0.28 °C per hour

- Pond 2 effective rate ~ 0.11–0.22 °C per hour

## Converting rate → afternoon temperature drop

Assuming sustained evaporation over ~3–4 peak hours:

### Pond 1

0.14–0.28×3–4=0.4–1.0∘C

### Pond 2

0.11–0.22×3–4=0.3–0.9∘C

Additive cooling of PS1&2:

(0.4–1.0)+(0.3–0.9)=0.9–1.9∘C

Together: ~0.9–1.9 °C local afternoon cooling potential (during average summer temperatures at case study site).

## ----------------------------------------------------------------------------------

## Estimating the impact of NWD bioswale evaporation on cooling of street during summer:

- Same street area: 2,400 m²

- Same mixing height: 5 m

- Same air volume: 12,000 m³

- Same realism factor: 20–40% of theoretical cooling persists

- Same summer afternoon duration: 3–4 hours

## 1. Convert annual evaporation volume → mass

1244.9 kL=1244.9 m3

Density of water ~1000 kg/m³:

m=1,244,900 kg per year

# 2. Convert mass → latent energy per year

Latent heat of vaporisation:

Lv=2.45×106 J/kg
E=1,244,900×2.45×106
E≈3.05×1012 J/year

# 3. Convert yearly energy → average continuous power

Seconds per year:

31,536,000 s
P=3.05×10123.1536×107
P≈96,700 W

# Bioswale latent cooling power

~97 kW (continuous average)

# 4. Convert power → theoretical temperature change rate

Heat capacity of local air volume (from earlier):

C=14.47×106 J/K
dTdt=96,70014,472,000=6.68×10-3 K/s

Convert to hourly:

6.68×10-3×3600=24.0 K/h

24 °C per hour is a purely theoretical rate and assumes that:

- All annual evaporation is occurring uniformly

- All latent cooling stayed inside the 12,000 m³ air volume

- No wind or heat inputs existed

But outdoors:

- Only a fraction of annual evaporation occurs in hot afternoons

- Only a fraction persists locally

# 5. Allocate evaporation to summer afternoons

A reasonable assumption:

- ~60% of annual evaporation occurs in summer

- ~30% of summer evaporation occurs during peak afternoon hours

0.6×0.3=0.18

~ 18% of annual evaporation contributes to afternoon cooling

Effective summer-afternoon power:

97 kW×0.18≈17.5 kW

# 6. Apply atmospheric persistence factor (20–40%)

17.5×0.2→3.5 kW
17.5×0.4→7.0 kW

# Effective bioswale cooling power during hot afternoons = 3.5-7.0kw

# 7. Convert to temperature reduction

dTdt=3,500–7,00014,472,000=0.00024–0.00048 K/s

Hourly:

0.86–1.73∘C per hour

NWD Bioswale cooling over 3–4 hours on a summer afternoon:

~2.6–6.9∘C

Combining Pond cooling and Bioswale cooling estimates for NWD:

~0.9–1.9 °C + ~2.6-6.9oC

= 3.5 oC -8.8 oC

## Combining bioswale and pond cooling with NWD shade estimate of 0.4oC (with 55% canopy coverage from small and medium-sized trees54)

=~0.9–1.9 °C + ~2.6-6.9oC + 0.4oC

=3.9-9.2 oC

Supplementary Data E. The calculations used to estimate the cooling potential of the WSUD scenario, including: A) Estimation of the impact of WSUD bioswale evaporation on cooling of street during summer; and B) Estimation of bioswale cooling combined with shade cooling estimate from urban heat modelling at the site.

## Estimation of the impact of WSUD bioswale evaporation on cooling of street during summer:

# 1. Convert annual evaporation → mass

726.8 kL=726.8 m3
m=726,800 kg/year

# 2. Convert mass → latent energy

Latent heat of vaporisation:

Lv=2.45×106 J/kg
E=726,800×2.45×106
E≈1.78×1012 J/year

# 3. Convert annual energy → average continuous power

Seconds per year:

31,536,000 s
P=1.78×10123.1536×107
P≈56.5 kW

# Average latent cooling power (annual mean)= 56.5kW

# 4. Allocate to summer afternoon conditions of site

Assumption:

- 60% of annual evaporation occurs in summer

- 30% of summer evaporation occurs during peak afternoon

0.6×0.3=0.18
56.5×0.18≈10.2 kW

# 5. Apply atmospheric persistence (20–40%)

10.2×0.2→2.0 kW
10.2×0.4→4.1 kW

# Effective bioswale cooling power (summer afternoon) = 2.0-4.1kW

# 6. Convert power → temperature reduction

Using same air heat capacity as NWD street calculations:

C=14.47×106 J/K
dTdt=2,000–4,10014.47×106
=0.00014–0.00028 K/s

Hourly estimate of temperature reduction = 0.50-1.02oC per hour 
Over a 3–4 hour hot afternoon = 1.5-4.1∘C

## WSUD bioswale cooling estimate:

For a street with 726.8 kL/year bioswale evaporation, expected local summer afternoon cooling:

~1.5 - 4 °C (assuming calm, sunny conditions, evaporation peak over 3-4hrs at site).

## Estimation of bioswale cooling combined with shade cooling estimate from urban heat modelling at the site

Report found: Maximum cooling effect estimated by heat modelling for BAU (9% canopy coverage54) is 0.06 oC:

~1.5 - 4 °C + 0.06

= ~2.1-4.06°C

Supplementary Data F. The master list of native vegetation species that was input into BAU, WSUD, and NWD designs in QGIS for analysis in the Vegetation Assessment Tool.

– Table too large, please see separate file.

Supplementary Data G– A more detailed methodology of how Vegetation Assessment Tool metrics were assessed.

### Density

The diversity of interstitial space within the foliage. Street design scored higher when the three density categories (closed, semi-open, open) were more represented and balanced across the site.

### Size

The proportion of plants within predetermined height categories at maturity. Scenarios scored higher when the four predetermined height categories (0-0.75m, 0.75-1.5m, 1.5-3m, 3m+) were more represented and balanced across the site.

### Texture

### Texture describes the roughness of the foliage. Scenarios scored higher when all three texture categories (fine, medium, coarse) were more represented and balanced across the site.

### Endemism

The proportion of species that are native to the country, not necessarily site, where the project will be realised. Scenarios scored higher for using more native and local vegetation due to its functions for other native species.

### Richness

The overall diversity (number of species) used on site. Richness is calculated from the average of a Shannon evenness score with a target baseline of 100 species per hectare. Evenness is shown comparing the total numbers of each species, as depicted in the size of the grey bars. A score close to zero indicates the dominance of a few species; 100/100 suggests a more balanced representation of species.

### Coverage

The relative proportion of vegetation at a 1m-cut height across the site through time. Coverage is a coarse proxy for measuring the presence of understory vegetation on site. Each bar represents a year in the specified growth simulation. It is preferable to not see any reductions in the bar heights throughout time as this indicates diminishing understory as the site vegetation matures. The grey portions of the bars represent the hypothetical maximums.

### Phenology

Phenology shows which plants are flowering in throughout the year. Each bar represents the relative proportions of plants that flower in each respective month. Scenarios score higher by providing more consistent resources for species through year.

### Connectivity

Connectivity is a measure of how much contiguous vegetation exists on the site. A connectivity value close to 100/100 indicates a high amount of contiguous or continuous vegetation throughout the site; a value closer to zero indicates very isolated patches of vegetation. This measure is a composite value based on the scores obtained at five heights (by default 0.3m, 1.0m, 1.7m, 2.4m, and 3.1m) which are indicated by the colour gradients in the bars (lower heights toward the inner circle). Connectivity changes as vegetation matures over time. Each bar represents a year in the specified growth simulation and grey portions in the bars represent the hypothetical maximums.

### Total score

This is the overall site score based on an unweighted average of all eight individual scores. Note that this score is out of 100 possible points.

# Supplementary Data H. Limitations of tools noted within method due to lack of integration.

- Modelling water quality benefits for low-pollutant flows in MUSICX

One limitation is related to modelling water quality benefits of NWD in MUSIC. Overall, the NWD scenario performed better than WSUD and BAU in water quality metrics, however, we note one discrepancy where WSUD appeared to remove more TSS than NWD. The high performance in pollutant removal of the WSUD scenario could be partially explained by Carex appressa’s high capacity for biofiltration (Szota et al., 2024). However, the NWD scenario also included C.appressa, with several other plants effective at nutrient removal, and a larger bioswale area. The close water quality outcomes between NWD and WSUD may instead be explained by a limitation we found in MUSICX’s ability to model extremely clean water (<12 mg/l TSS). After water was filtered through the bioswales of the NWD street, it was conveyed into ponds for further biofiltration. However, MUSICX is typically used to simulate “dirty” stormwater treatment and uses an equation ((C~out~ - C*) / (C~in~ - C*) = e –k/q) that assumes a minimum TSS outflow concentration of 12 mg/l. Therefore, when inflow rates dropped below 12 mg/L, MUSICX was unable to assess water quality improvement.

We further tested this hypothesis by removing the ponds from our NWD model and found that the TSS removal of NWD increased (93.4% to 96.8%) after removing the ponds - exceeding the WSUD TSS reduction of 96.6%. Considering the available literature on the typical benefits of ponds for biofiltration (Liu et al., 2024; Lusk et al., 2025; Shen et al., 2025), we expect that our model underestimated the water quality benefits of the NWD street ponds due to this rounding-up error. Therefore, further tool development could better incorporate scenarios that require lower pollutant concentrations for pollutant-sensitive species.

- Modelling nuanced biodiversity impacts

Another limitation of MUSICX was that it could model the impacts of only a fixed number and density of species within treatment types. Based on the literature available, our models assumed that bioswales in WSUD and NWD functioned equally well at removing nutrients by both having plants ‘effective at nutrient removal’ (Supplementary Table 6). However, this tool’s limitations underrepresented nuanced impacts arising from higher plant density and diversity in NWD bioswales. Furthermore, we were unable to model the BSUD scenario in MUSIC because the number and density of species could not be represented without a WSUD treatment node.

While many gaps in the knowledgebase still exist, an increasing number of experimental studies identify the nutrient or pollutant removal effectiveness of different plant species (Bratieres et al., 2008; Read et al., 2008; Payne et al., 2018; Sete et al., 2020). To improve the integration of biodiversity into a tool such as MUSICX, these evidence-based estimates of species-specific nutrient-removal and filtration capacity could be included as a customisable feature within verge, bioswale, pond or wetland nodes. Furthermore, integrated tools could incorporate other trait variations affected by species diversity and density. For example, new tools could incorporate impacts on water use, drought tolerance, evapotranspiration (De Boeck et al., 2006; Lübbe et al., 2016) or biodiversity-water related processes such as nitrogen fixation (Prudent et al., 2016), soil-water storage (Kammer et al., 2013), or carbon sequestration (Yan et al., 2022). This may also help inform the maintenance needs of different assets over time and how management can balance trade-offs between, for example, ecological establishment and asset drainage needs (Mullaly, 2019). Furthermore, integrated design and planning could be better supported by more software packages specifically designed for the evaluation of urban biodiversity-water outcomes (Hoegh-Guldberg et al., In press).